Drain Boss

Safety data sheet

According to regulation (Ec) no. 1907/2006 (REACH) with it amendment regulation (EU) 2015/830 issue date 8/4/2020 version: 0.0

SECTION 1: Identification of the substance/mixture and of the company/undertaking

1.1. Product identifier

Product name	: Drain Boss

Product code	: Db 01

1.2. Relevant identified uses of the substance or mixture and uses advised against

## Relevant identified uses

Main use category	: Intermediate,Processing aid,Surface treatment,Catalyst,Acidifier

## Uses advised against

No additional information available

1.3. Details of the supplier of the safety data sheet

## Supplier

## Dargan Tools Limited

## Dolmen Business Park

## Tullow road

## Carlow

## (059) 9142855

1.4. Emergency telephone number

Emergency number	: +353 (086) 4067748

Office Hours: Monday to Thursday 9.00am- 5.30pm, Friday 9.00am- 4.30pm

SECTION 2: Hazards identification

2.1. Classification of the substance or mixture

## Classification according to Regulation (EC) No. 1272/2008 [CLP]

Skin corrosion/irritation, Category 1A	H314 Full text of H statements : see section 16

## Adverse physicochemical, human health and environmental effects

To our knowledge, this product does not present any particular risk, provided it is handled in accordance with good occupational hygiene and safety practice.

2.2. Label elements

## Labelling according to Regulation (EC) No. 1272/2008 [CLP]

Hazard pictograms (CLP)	:

GHS05

Signal word (CLP)	: Danger

Hazard statements (CLP)	: H314 - Causes severe skin burns and eye damage.

Precautionary statements (CLP)	: P202 - Do not handle until all safety precautions have been read and understood.

P280 - Wear protective clothing/eye protection/face protection. P303+P361+P353+P310 - IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water/shower.. Immediately call a POISON CENTER, a doctor. P305+P351+P338+P310 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. Immediately call a doctor, a POISON CENTER.

P338 - Remove contact lenses, if present and easy to do. Continue rinsing. P405 - Store locked up.

2.3. Other hazards

Other hazards not contributing to the classification	: Reacts with water (moisture): release of heat.

SECTION 3: Composition/information on ingredients

3.1. Substances

Not applicable

3.2. Mixtures

| Name | Product identifier | % | Classification according to Regulation (EC) No. 1272/2008 [CLP] |
| --- | --- | --- | --- |
| Drain Boss | (CAS-No.) 7664-93-9 (EC-No.) 231-639-5 (REACH-no) 01-2119958838-20 | 94 - 97 | Skin Corr. 1, H314 |

| Specific concentration limits: |  |  |
| --- | --- | --- |
| Name | Product identifier | Specific concentration limits |
| Drain Boss | (CAS-No.) 7664-93-9 (EC-No.) 231-639-5 (REACH-no) 01-2119958838-20 | ( 5 ≤C < 15) Skin Irrit. 2, H315 ( 5 ≤C < 15) Eye Irrit. 2, H319 ( 15 ≤C < 100) Skin Corr. 1A, H314 |

Full text of H-statements: see section 16

SECTION 4: First aid measures

4.1. Description of first aid measures

First-aid measures after inhalation	: Remove person to fresh air and keep comfortable for breathing. Call a physician if

symptoms occur.

First-aid measures after skin contact	: Wash skin with plenty of water. Take off immediately all contaminated clothing and wash it

before reuse. If skin irritation occurs: Get medical advice/attention.

First-aid measures after eye contact	: Consult an eye specialist. In case of contact, immediately flush eye with plenty of water for

at least 15 minutes.

First-aid measures after ingestion	: Drinking water. Rinse mouth out with water. Do NOT induce vomiting. Get medical

advice/attention.

4.2. Most important symptoms and effects, both acute and delayed

Symptoms/effects	: Causes severe burns.

4.3. Indication of any immediate medical attention and special treatment needed

Treat symptomatically. Rinse with plenty of water.

SECTION 5: Firefighting measures

5.1. Extinguishing media

Suitable extinguishing media	: Dry powder. Sand.

Unsuitable extinguishing media	: Strong water jet.

5.2. Special hazards arising from the substance or mixture

Hazardous decomposition products in case of fire	: Toxic fumes may be released.

5.3. Advice for firefighters

Protection during firefighting	: Do not attempt to take action without suitable protective equipment. Self-contained

breathing apparatus. Complete protective clothing.

SECTION 6: Accidental release measures

6.1. Personal precautions, protective equipment and emergency procedures

## For non-emergency personnel

Emergency procedures	: Ventilate spillage area. Keep upwind. Avoid contact with skin and eyes.

## For emergency responders

Protective equipment	: Do not attempt to take action without suitable protective equipment. For further information refer to section 8: "Exposure controls/personal protection".

6.2. Environmental precautions

Avoid release to the environment.

6.3. Methods and material for containment and cleaning up

For containment	: Liquid spill: neutralize with powdered limestone or sodium bicarbonate. Rinse with plenty of water.

Other information	: Only qualified personnel equipped with suitable protective equipment may intervene.

Dispose of materials or solid residues at an authorized site.

6.4. Reference to other sections

For further information refer to section 13.

SECTION 7: Handling and storage

7.1. Precautions for safe handling

Precautions for safe handling	: Ensure good ventilation of the work station. Wear personal protective equipment. In case of

inadequate ventilation wear respiratory protection. Emergency eye wash fountains should be available in the immediate vicinity of any potential exposure.

Hygiene measures	: Do not eat, drink or smoke when using this product. Always wash hands after handling the product.

7.2. Conditions for safe storage, including any incompatibilities

Storage conditions	: Store in a well-ventilated place. Keep cool. Store in original container. Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking.

Incompatible materials	: Bases. Flammable materials. Chlorates. Nitrates. Oxidising substances.

7.3. Specific end use(s)

No additional information available

SECTION 8: Exposure controls/personal protection

8.1. Control parameters

| Drain Boss |  |
| --- | --- |
| Ireland - Occupational Exposure Limits |  |
| OEL (8 hours ref) (mg/m³) | 1 mg/m³ Where no specific short-term exposure limit is listed, a figure three times the long-term exposure limit value should be used |

| Drain Boss |  |
| --- | --- |
| Ireland - Occupational Exposure Limits |  |
| OEL (8 hours ref) (mg/m³) | 1 mg/m³ |

# 8.2. Exposure controls

## Appropriate engineering controls:

Ensure good ventilation of the work station.

| Hand protection: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Protective gloves |  |  |  |  |  |
| Type | Material | Permeation | Thickness (mm) | Penetration | Standard |
| Reusable gloves | Butyl rubber | 6 (> 480 minutes) |  |  |  |
| Reusable gloves | Polyethylene | 6 (> 480 minutes) |  |  |  |
| Reusable gloves | Viton | 5 (> 240 minutes) |  |  |  |

tightly fitting safety goggles

Eye protection:

Wear suitable protective clothing. Protect eyes, face and skin from liquid splashes.

Skin and body protection:

In case of insufficient ventilation, wear suitable respiratory equipment

Respiratory protection:

## Personal protective equipment symbol(s):

Environmental exposure controls:

Avoid release to the environment.

## Other information:

Avoid contact with skin and eyes. Wash hands before breaks and after work. Emergency eye wash fountains and safety showers should be available in the immediate vicinity of any potential exposure.

SECTION 9: Physical and chemical properties

# 9.1. Information on basic physical and chemical properties

Physical state	: Liquid

Appearance	: Oily.

Colour	: Colourless. clear.

Odour	: No data available

Odour threshold	: No data available

pH	: < 1

Relative evaporation rate (butylacetate=1)	: No data available Melting point	: No data available

Freezing point	: ≈ -1 °C 98% H2SO4

Boiling point	: ≈ 327 °C 98% H2SO4

Flash point	: No data available

Auto-ignition temperature	: No data available

Decomposition temperature	: > 300 °C

Flammability (solid, gas)	: No data available

Vapour pressure	: 0.0121 Pa @ 20°C, 98% H2SO4

Relative vapour density at 20 °C	: No data available

Relative density	: No data available

Density	: 1.8 g/cm³ 98% H2SO4

Solubility	: Soluble in water.

Partition coefficient n-octanol/water (Log Pow)	: No data available Viscosity, kinematic	: ≈ 16.667 mm²/s

Viscosity, dynamic	: ≈ 30 mPa·s @ 20°C, 98% H2SO4

Explosive properties	: No data available

Oxidising properties	: None.

Explosive limits	: No data available

9.2. Other information

No additional information available

SECTION 10: Stability and reactivity

10.1. Reactivity

Exothermic reaction with water. Do not add water to product. Liquid splashes may occur.

10.2. Chemical stability

Stable under normal conditions.

10.3. Possibility of hazardous reactions

Contact with metals produces hydrogen gas which may form explosive mixtures with air.

10.4. Conditions to avoid

High temperature.

10.5. Incompatible materials

Flammable materials. Bases. Nitrates. Oxidizing agent.

10.6. Hazardous decomposition products

Sulfur dioxide and hydrogen chloride (acidic gases).

SECTION 11: Toxicological information

11.1. Information on toxicological effects

Acute toxicity (oral)	: Not classified

Acute toxicity (dermal)	: Not classified

Acute toxicity (inhalation)	: Not classified

| Drain Boss |  |
| --- | --- |
| LD50 oral rat | 2140 mg/kg bodyweight Animal: rat, 95% CL: 1540 - 2990 |
| LC50 inhalation rat (mg/l) | 0.375 mg/l air Animal: rat, Guideline: OECD Guideline 403 (Acute Inhalation Toxicity) |

Skin corrosion/irritation	: Causes severe skin burns. pH: < 1

Serious eye damage/irritation	: Assumed to cause serious eye damage pH: < 1

Respiratory or skin sensitisation	: No sensitisation responses were observed

Germ cell mutagenicity	: Not classified

Carcinogenicity	: Not classified

Reproductive toxicity	: Not classified

STOT-single exposure	: Not classified

STOT-repeated exposure	: Not classified

Aspiration hazard	: Not classified

| Drain Boss |  |
| --- | --- |
| Viscosity, kinematic | ≈ 16.667 mm²/s |

SECTION 12: Ecological information

# Toxicity

Ecology - general	: The product is not considered harmful to aquatic organisms nor to cause long-term adverse effects in the environment.

Hazardous to the aquatic environment, short-term (acute)

Hazardous to the aquatic environment, long-term (chronic)

Not rapidly degradable

: Not classified

: Not classified

| Drain Boss |  |
| --- | --- |
| EC50 Daphnia 1 | > 100 mg/l Test organisms (species): Daphnia magna |
| EC50 72h algae (1) | > 100 mg/l Test organisms (species): Desmodesmus subspicatus (previous name: Scenedesmus subspicatus) |
| NOEC (chronic) | 0.15 mg/l Test organisms (species): other:Tanytarsus dissimilis |
| NOEC chronic fish | 0.31 mg/l Test organisms (species): Salvelinus fontinalis |

  - Persistence and degradability

| Drain Boss |  |
| --- | --- |
| Persistence and degradability | Not applicable. Inorganic Particulate Matter. |

  - Bioaccumulative potential

| Drain Boss |  |
| --- | --- |
| Bioaccumulative potential | No bioaccumulation. |

  - Mobility in soil

| Drain Boss |  |
| --- | --- |
| Ecology - soil | To protect soil and water living organisms, application of this product is restricted to indoor areas or areas under roof with a hard standing, where drain away to soil or surface water can be prevented. |

  - Results of PBT and vPvB assessment

| Drain Boss |  |
| --- | --- |
| Results of PBT assessment | This substance is not considered to be persistent, bioaccumulating and toxic (PBT). This substance is not considered to be very persistent and very bioaccumulating (vPvB). |

  - Other adverse effects

No additional information available

SECTION 13: Disposal considerations

# 13.1. Waste treatment methods

Waste treatment methods	: After neutralizing, spray with large amounts of water. Dispose of contents/container in

accordance with licensed collector’s sorting instructions. Must follow special treatment according to local regulation.

Product/Packaging disposal recommendations	: Clean using water and a detergent. Refer to manufacturer/supplier for information on

recovery/recycling.

SECTION 14: Transport information

In accordance with ADR / RID / IMDG / IATA / ADN

| ADR | IMDG | IATA | ADN | RID |
| --- | --- | --- | --- | --- |
| 14.1. UN number |  |  |  |  |
| UN 1830 | UN 1830 | Not applicable | UN 1830 | UN 1830 |
| 14.2. UN proper shipping name |  |  |  |  |
| SULPHURIC ACID | SULPHURIC ACID | Not applicable | SULPHURIC ACID | SULPHURIC ACID |
| Transport document description |  |  |  |  |
| UN 1830 SULPHURIC ACID, 8, II, (E) | UN 1830 SULPHURIC ACID, 8, II | Not applicable | UN 1830 SULPHURIC ACID, 8, II | UN 1830 SULPHURIC ACID, 8, II |
| 14.3. Transport hazard class(es) |  |  |  |  |
| 8 | 8 | Not applicable | 8 | 8 |
|  |  | Not applicable |  |  |
| 14.4. Packing group |  |  |  |  |
| II | II | Not applicable | II | II |
| 14.5. Environmental hazards |  |  |  |  |
| Dangerous for the environment : No | Dangerous for the environment : No Marine pollutant : No | Not applicable | Dangerous for the environment : No | Dangerous for the environment : No |
| No supplementary information available |  |  |  |  |

# 14.6. Special precautions for user

## Overland transport

Classification code (ADR)	: C1

Limited quantities (ADR)	: 1l

Excepted quantities (ADR)	: E2

Packing instructions (ADR)	: P001, IBC02

Mixed packing provisions (ADR)	: MP15 Portable tank and bulk container instructions (ADR) : T8

Portable tank and bulk container special provisions (ADR)

: TP2

Tank code (ADR)	: L4BN

Tank special provisions (ADR)	: TU42

Vehicle for tank carriage	: AT

Transport category (ADR)	: 2 Hazard identification number (Kemler No.)	: 80

Orange plates	:

Tunnel restriction code (ADR)	: E

EAC code	: 2P

## Transport by sea

Packing instructions (IMDG)	: P001

IBC packing instructions (IMDG)	: IBC02

IBC special provisions (IMDG)	: B20

Tank instructions (IMDG)	: T8

Tank special provisions (IMDG)	: TP2

EmS-No. (Fire)	: F-A

EmS-No. (Spillage)	: S-B

Stowage category (IMDG)	: C

Stowage and handling (IMDG)	: SW15

Properties and observations (IMDG)	: Colourless, oily liquid, mixture over 1.41 up to 1.84 relative density. In the presence of

moisture, highly corrosive to most metals. Causes burns to skin, eyes and mucous membranes.

## Air transport

Not applicable

## Inland waterway transport

Classification code (ADN)	: C1

Limited quantities (ADN)	: 1 L

Excepted quantities (ADN)	: E2

Carriage permitted (ADN)	: T

Equipment required (ADN)	: PP, EP

Number of blue cones/lights (ADN)	: 0

## Rail transport

Classification code (RID)	: C1

Limited quantities (RID)	: 1L

Excepted quantities (RID)	: E2

Packing instructions (RID)	: P001, IBC02

Mixed packing provisions (RID)	: MP15 Portable tank and bulk container instructions (RID)	: T8

Portable tank and bulk container special provisions (RID)

: TP2

Tank codes for RID tanks (RID)	: L4BN

Special provisions for RID tanks (RID)	: TU42

Transport category (RID)	: 2

Colis express (express parcels) (RID)	: CE6

Hazard identification number (RID)	: 80

14.7. Transport in bulk according to Annex II of Marpol and the IBC Code

Not applicable

SECTION 15: Regulatory information

15.1. Safety, health and environmental regulations/legislation specific for the substance or mixture

## EU-Regulations

Contains no REACH substances with Annex XVII restrictions Contains no substance on the REACH candidate list Contains no REACH Annex XIV substances

Contains no substance subject to Regulation (EU) No 649/2012 of the European Parliament and of the Council of 4 July 2012 concerning the export and import of hazardous chemicals.

Contains no substance subject to Regulation (EU) No 2019/1021 of the European Parliament and of the Council of 20 June 2019 on persistent organic pollutants

## National regulations Germany

Employment restrictions	: Observe restrictions according Act on the Protection of Working Mothers (MuSchG) Observe restrictions according Act on the Protection of Young People in Employment (JArbSchG)

Water hazard class (WGK)	: WGK 1, Slightly hazardous to water (Classification according to AwSV, Annex 1) Hazardous Incident Ordinance (12. BImSchV)	: Is not subject of the Hazardous Incident Ordinance (12. BImSchV)

## Netherlands

SZW-lijst van kankerverwekkende stoffen	: Sulphuric acid is listed

SZW-lijst van mutagene stoffen	: None of the components are listed

NIET-limitatieve lijst van voor de voortplanting giftige stoffen – Borstvoeding

NIET-limitatieve lijst van voor de voortplanting giftige stoffen – Vruchtbaarheid

NIET-limitatieve lijst van voor de voortplanting giftige stoffen – Ontwikkeling

## Denmark

: None of the components are listed

: None of the components are listed

: None of the components are listed

Danish National Regulations	: Young people below the age of 18 years are not allowed to use the product

# 15.2. Chemical safety assessment

No chemical safety assessment has been carried out

SECTION 16: Other information

| Full text of H- and EUH-statements: |  |
| --- | --- |
| Eye Irrit. 2 | Serious eye damage/eye irritation, Category 2 |
| Skin Corr. 1 | Skin corrosion/irritation, Category 1 |
| Skin Corr. 1A | Skin corrosion/irritation, Category 1A |
| Skin Irrit. 2 | Skin corrosion/irritation, Category 2 |
| H314 | Causes severe skin burns and eye damage. |
| H315 | Causes skin irritation. |
| H319 | Causes serious eye irritation. |

SDS EU (REACH Annex II)

This information is based on our current knowledge and is intended to describe the product for the purposes of health, safety and environmental requirements only. It should not therefore be construed as guaranteeing any specific property of the product.

"Emergency medical information: (seven days) contact National Poisons Information Centre, Beaumont Hospital, Dublin 9 DOV2NO, Ireland.

Members of the public Number (8 am-10 pm): "+353 (0)1 809 2166 ".

Healthcare professional telephone Number (24hrs): "+353 (0)1 809 2566"