Boss Devil

Safety Data Sheet

according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830 Issue date: 13/03/2020 Version: 0.0

SECTION 1: Identification of the substance/mixture and of the company/undertaking

1.1. Product identifier

Product form	: Mixture

Product name	: Boss Devil

Product code	: MB01

1.2. Relevant identified uses of the substance or mixture and uses advised against

## Relevant identified uses

Main use category	: Industrial use

Use of the substance/mixture	: Detergents

## Uses advised against

No additional information available

1.3. Details of the supplier of the safety data sheet

## Supplier

## Dargan Tools Limited

## Dolmen Business Park

## Tullow road

## Carlow

## (059) 9142855

1.4. Emergency telephone number

Emergency number	: +353 (086) 4067748

Office Hours: Monday to Thursday 9.00am- 5.30pm, Friday 9.00am- 4.30pm

SECTION 2: Hazards identification

2.1. Classification of the substance or mixture

## Classification according to Regulation (EC) No. 1272/2008 [CLP]

Acute toxicity (oral), Category 4	H302

Skin corrosion/irritation, Category 1, Sub-Category 1B	H314

Serious eye damage/eye irritation, Category 1	H318 Hazardous to the aquatic environment — Acute Hazard, Category 1	H400 Hazardous to the aquatic environment — Chronic Hazard, Category 1     H410 Full text of H statements : see section 16

## Adverse physicochemical, human health and environmental effects

May cause damage to organs through prolonged or repeated exposure. Harmful if swallowed. Causes severe skin burns and eye damage. Causes serious eye damage. Very toxic to aquatic life with long lasting effects.

2.2. Label elements

## Labelling according to Regulation (EC) No. 1272/2008 [CLP]

Hazard pictograms (CLP)	:

GHS05	GHS07	GHS09

Signal word (CLP)	: Danger

Contains	: Quaternary ammonium compounds, benzyl-C12-14 (even-numbered)-alkyldimethyl, chlorides

Hazard statements (CLP)	: H302 - Harmful if swallowed.

H314 - Causes severe skin burns and eye damage. H410 - Very toxic to aquatic life with long lasting effects.

Precautionary statements (CLP)	: P273 - Avoid release to the environment.

P280 - Wear protective clothing, eye protection, face protection, protective gloves. P301+P312 - IF SWALLOWED: Call doctor, a POISON CENTER if you feel unwell. P302+P352 - IF ON SKIN: Wash with plenty of water.

P305+P351+P338 - IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

P501 - Dispose of an approved waste disposal plant to a licensed hazardous-waste disposal contractor or collection site except for empty clean containers which can be disposed of as non-hazardous waste.

2.3. Other hazards

No additional information available

SECTION 3: Composition/information on ingredients

3.1. Substances

Not applicable

3.2. Mixtures

| Name | Product identifier | % | Classification according to Regulation (EC) No. 1272/2008 [CLP] |
| --- | --- | --- | --- |
| Quaternary ammonium compounds, benzyl-C12-14 (even-numbered)-alkyldimethyl, chlorides | EC-No.: 939-350-2 | 50 - < 60 | Acute Tox. 4 (Oral), H302 Skin Corr. 1B, H314 Eye Dam. 1, H318 Aquatic Acute 1, H400 (M=10) Aquatic Chronic 1, H410 |

Full text of H-statements: see section 16

SECTION 4: First aid measures

4.1. Description of first aid measures

First-aid measures general	: Call a physician immediately.

First-aid measures after inhalation	: Remove person to fresh air and keep comfortable for breathing.

First-aid measures after skin contact	: Rinse skin with water/shower. Take off immediately all contaminated clothing. Call a

physician immediately.

First-aid measures after eye contact	: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy

to do. Continue rinsing. Call a physician immediately.

First-aid measures after ingestion	: Rinse mouth. Do not induce vomiting. Call a physician immediately.

4.2. Most important symptoms and effects, both acute and delayed

Symptoms/effects after skin contact	: Burns.

Symptoms/effects after eye contact	: Serious damage to eyes.

Symptoms/effects after ingestion	: Burns.

4.3. Indication of any immediate medical attention and special treatment needed

Treat symptomatically.

SECTION 5: Firefighting measures

5.1. Extinguishing media

Suitable extinguishing media	: Use extinguishing media appropriate for surrounding fire.

Unsuitable extinguishing media	: Strong water jet.

5.2. Special hazards arising from the substance or mixture

Hazardous decomposition products in case of fire	: Toxic fumes may be released. hydrogen chloride. Nitrogen oxides. Hydrogen cyanide.

5.3. Advice for firefighters

Protection during firefighting	: Do not attempt to take action without suitable protective equipment. Self-contained

breathing apparatus. Complete protective clothing.

SECTION 6: Accidental release measures

6.1. Personal precautions, protective equipment and emergency procedures

## For non-emergency personnel

Emergency procedures	: Ventilate spillage area. Do not breathe fume, mist, vapours. Avoid contact with skin and eyes.

## For emergency responders

Protective equipment	: Do not attempt to take action without suitable protective equipment. For further information refer to section 8: "Exposure controls/personal protection".

6.2. Environmental precautions

Avoid release to the environment. Do not allow to enter drains or water courses. Notify authorities if liquid enters sewers or public waters.

6.3. Methods and material for containment and cleaning up

For containment	: Collect spillage.

Methods for cleaning up	: Take up liquid spill into absorbent material.

Other information	: Dispose of materials or solid residues at an authorized site.

6.4. Reference to other sections

For further information refer to section 13.

SECTION 7: Handling and storage

7.1. Precautions for safe handling

Precautions for safe handling	: Ensure good ventilation of the work station. Do not breathe fume, mist, vapours. Avoid

contact with skin and eyes. Wear personal protective equipment.

Hygiene measures	: Wash contaminated clothing before reuse. Do not eat, drink or smoke when using this product. Always wash hands after handling the product.

7.2. Conditions for safe storage, including any incompatibilities

Storage conditions	: Store locked up. Store in a well-ventilated place. Keep cool.

Information on mixed storage	: Keep away from food, drink and animal feedingstuffs.

Special rules on packaging	: Keep only in original container. Store in a closed container.

7.3. Specific end use(s)

No additional information available

SECTION 8: Exposure controls/personal protection

8.1. Control parameters

## National occupational exposure and biological limit values

No additional information available

## Recommended monitoring procedures

No additional information available

## Air contaminants formed

No additional information available

## DNEL and PNEC

No additional information available

## Control banding

No additional information available

8.2. Exposure controls

## Appropriate engineering controls Appropriate engineering controls:

Ensure good ventilation of the work station. Emergency eye wash fountains and safety showers should be available in the immediate vicinity of any potential exposure.

## Personal protection equipment Personal protective equipment symbol(s):

      - Eye and face protection Eye protection:

tightly fitting safety goggles

## Skin protection Skin and body protection:

Wear suitable protective clothing. Safety boots

## Hand protection:

Protective gloves made of PVC

## Respiratory protection Respiratory protection:

In case of insufficient ventilation, wear suitable respiratory equipment

## Thermal hazards

No additional information available

## Environmental exposure controls Environmental exposure controls:

Avoid release to the environment.

SECTION 9: Physical and chemical properties

9.1. Information on basic physical and chemical properties

Physical state	: Liquid

Colour	: colourless to slightly yellow.

Odour	: characteristic.

Odour threshold	: No data available

pH	: No data available

Relative evaporation rate (butylacetate=1)	: No data available Melting point	: Not applicable

Freezing point	: No data available

Boiling point	: > 100 °C

Flash point	: > 100 °C

Auto-ignition temperature	: No data available

Decomposition temperature	: No data available

Flammability (solid, gas)	: Not applicable

Vapour pressure	: No data available

Relative vapour density at 20 °C	: No data available

Relative density	: No data available

Density	: 0.99 – 1 g/cm³ @ 20°C

Solubility	: Miscible with water.

Partition coefficient n-octanol/water (Log Pow)	: No data available Viscosity, kinematic	: No data available

Viscosity, dynamic	: No data available

Explosive properties	: No data available

Oxidising properties	: No data available

Explosive limits	: No data available

9.2. Other information

No additional information available

SECTION 10: Stability and reactivity

10.1. Reactivity

The product is non-reactive under normal conditions of use, storage and transport.

10.2. Chemical stability

Stable under normal conditions.

10.3. Possibility of hazardous reactions

No dangerous reactions known under normal conditions of use.

10.4. Conditions to avoid

To avoid thermal decomposition, do not overheat.

10.5. Incompatible materials

No additional information available

10.6. Hazardous decomposition products

Nitrogen oxides. Carbon oxides (CO, CO2).

SECTION 11: Toxicological information

11.1. Information on toxicological effects

Acute toxicity (oral)	: Harmful if swallowed.

Acute toxicity (dermal)	: Not classified

Acute toxicity (inhalation)	: Not classified

| Boss Devil |  |
| --- | --- |
| ATE CLP (oral) | 662.5 mg/kg bodyweight |

| Quaternary ammonium compounds, benzyl-C12-14 (even-numbered)-alkyldimethyl, chlorides |  |
| --- | --- |
| LD50 oral rat | 397.5 mg/kg |
| LD50 dermal rabbit | 3412.5 mg/kg bodyweight Animal: rabbit, Guideline: EPA OPPTS 870.1200 (Acute Dermal Toxicity) |

Skin corrosion/irritation	: Causes severe skin burns.

Serious eye damage/irritation	: Causes serious eye damage.

Respiratory or skin sensitisation	: Not classified

Germ cell mutagenicity	: Not classified

Carcinogenicity	: Not classified

Reproductive toxicity	: Not classified

STOT-single exposure	: Not classified

STOT-repeated exposure	: Not classified

| Quaternary ammonium compounds, benzyl-C12-14 (even-numbered)-alkyldimethyl, chlorides |  |
| --- | --- |
| NOAEL (subchronic, oral, animal/male, 90 days) | 50 mg/kg bodyweight Animal: dog, Animal sex: male, Guideline: OECD Guideline 409 (Repeated Dose 90-Day Oral Toxicity in Non-Rodents) |
| NOAEL (subchronic, oral, animal/female, 90 days) | 45 mg/kg bodyweight Animal: dog, Animal sex: female, Guideline: OECD Guideline 409 (Repeated Dose 90-Day Oral Toxicity in Non-Rodents) |

Aspiration hazard	: Not classified

SECTION 12: Ecological information

12.1. Toxicity

Ecology - general	: Very toxic to aquatic life with long lasting effects.

Hazardous to the aquatic environment, short-term (acute)

Hazardous to the aquatic environment, long-term (chronic)

Not rapidly degradable

: Very toxic to aquatic life.

: Very toxic to aquatic life with long lasting effects.

| Quaternary ammonium compounds, benzyl-C12-14 (even-numbered)-alkyldimethyl, chlorides |  |
| --- | --- |
| LC50 - Fish [1] | 1.7 mg/l Test organisms (species): Cyprinodon variegatus |
| LC50 - Fish [2] | 1.28 mg/l Test organisms (species): Cyprinodon variegatus |
| EC50 72h - Algae [1] | 0.26 mg/l Test organisms (species): Skeletonema costatum |

12.2. Persistence and degradability

No additional information available

12.3. Bioaccumulative potential

No additional information available

12.4. Mobility in soil

No additional information available

12.5. Results of PBT and vPvB assessment

No additional information available

12.6. Other adverse effects

No additional information available

SECTION 13: Disposal considerations

# 13.1. Waste treatment methods

Waste treatment methods	: Do not allow to enter drains or water courses. Contact waste disposal services. Dispose of

contents/container in accordance with licensed collector’s sorting instructions. Must follow special treatment according to local regulation. Dispose as hazardous waste.

Product/Packaging disposal recommendations	: Completely empty the packaging prior to decontamination. Clean using water and a

detergent. Refer to manufacturer/supplier for information on recovery/recycling.

SECTION 14: Transport information

In accordance with ADR / IMDG / IATA / ADN / RID

| ADR | IMDG | IATA | ADN | RID |
| --- | --- | --- | --- | --- |
| 14.1. UN number |  |  |  |  |
| UN 1760 | UN 1760 | UN 1760 | UN 1760 | UN 1760 |
| 14.2. UN proper shipping name |  |  |  |  |
| CORROSIVE LIQUID, N.O.S. (CONTAINS : Quaternary ammonium compounds, benzyl-C12-14 (even-numbered)- alkyldimethyl, chlorides) | CORROSIVE LIQUID, N.O.S. (CONTAINS : Quaternary ammonium compounds, benzyl-C12-14 (even-numbered)- alkyldimethyl, chlorides) | Corrosive liquid, n.o.s. (CONTAINS : Quaternary ammonium compounds, benzyl-C12-14 (even- numbered)-alkyldimethyl, chlorides) | CORROSIVE LIQUID, N.O.S. (CONTAINS : Quaternary ammonium compounds, benzyl-C12-14 (even-numbered)- alkyldimethyl, chlorides) | CORROSIVE LIQUID, N.O.S. (CONTAINS : Quaternary ammonium compounds, benzyl-C12-14 (even-numbered)- alkyldimethyl, chlorides) |
| Transport document description |  |  |  |  |
| UN 1760 CORROSIVE LIQUID, N.O.S. (CONTAINS : Quaternary ammonium compounds, benzyl-C12-14 (even- numbered)-alkyldimethyl, chlorides), 8, II, (E), ENVIRONMENTALLY HAZARDOUS | UN 1760 CORROSIVE LIQUID, N.O.S. (CONTAINS : Quaternary ammonium compounds, benzyl-C12-14 (even- numbered)-alkyldimethyl, chlorides), 8, II, MARINE POLLUTANT/ENVIRONME NTALLY HAZARDOUS | UN 1760 Corrosive liquid, n.o.s. (CONTAINS : Quaternary ammonium compounds, benzyl-C12-14 (even-numbered)- alkyldimethyl, chlorides), 8, II, ENVIRONMENTALLY HAZARDOUS | UN 1760 CORROSIVE LIQUID, N.O.S. (CONTAINS : Quaternary ammonium compounds, benzyl-C12-14 (even- numbered)-alkyldimethyl, chlorides), 8, II, ENVIRONMENTALLY HAZARDOUS | UN 1760 CORROSIVE LIQUID, N.O.S. (CONTAINS : Quaternary ammonium compounds, benzyl-C12-14 (even- numbered)-alkyldimethyl, chlorides), 8, II, ENVIRONMENTALLY HAZARDOUS |
| 14.3. Transport hazard class(es) |  |  |  |  |
| 8 | 8 | 8 | 8 | 8 |
| 14.4. Packing group |  |  |  |  |
| II | II | II | II | II |
| 14.5. Environmental hazards |  |  |  |  |
| Dangerous for the environment: Yes | Dangerous for the environment: Yes Marine pollutant: Yes | Dangerous for the environment: Yes | Dangerous for the environment: Yes | Dangerous for the environment: Yes |
| No supplementary information available |  |  |  |  |

# Special precautions for user

## Overland transport

Classification code (ADR)	: C9

Special provisions (ADR)	: 274

Limited quantities (ADR)	: 1l

Excepted quantities (ADR)	: E2

Packing instructions (ADR)	: P001, IBC02

Mixed packing provisions (ADR)	: MP15 Portable tank and bulk container instructions (ADR) : T11

Portable tank and bulk container special provisions (ADR)

: TP2, TP27

Tank code (ADR)	: L4BN

Vehicle for tank carriage	: AT

Transport category (ADR)	: 2 Hazard identification number (Kemler No.)	: 80 Orange plates	:

Tunnel restriction code (ADR)	: E

EAC code	: 2X

APP code	: B

## Transport by sea

Special provisions (IMDG)	: 274

Packing instructions (IMDG)	: P001

IBC packing instructions (IMDG)	: IBC02

Tank instructions (IMDG)	: T11

Tank special provisions (IMDG)	: TP2, TP27

EmS-No. (Fire)	: F-A

EmS-No. (Spillage)	: S-B

Stowage category (IMDG)	: B

Stowage and handling (IMDG)	: SW2

Properties and observations (IMDG)	: Causes burns to skin, eyes and mucous membranes.

## Air transport

PCA Excepted quantities (IATA)	: E2

PCA Limited quantities (IATA)	: Y840 PCA limited quantity max net quantity (IATA)	: 0.5L PCA packing instructions (IATA)	: 851

PCA max net quantity (IATA)	: 1L

CAO packing instructions (IATA)	: 855

CAO max net quantity (IATA)	: 30L

Special provisions (IATA)	: A3, A803

ERG code (IATA)	: 8L

## Inland waterway transport

Classification code (ADN)	: C9

Special provisions (ADN)	: 274

Limited quantities (ADN)	: 1 L

Excepted quantities (ADN)	: E2

Equipment required (ADN)	: PP, EP

Number of blue cones/lights (ADN)	: 0

## Rail transport

Classification code (RID)	: C9

Special provisions (RID)	: 274

Limited quantities (RID)	: 1L

Excepted quantities (RID)	: E2

Packing instructions (RID)	: P001, IBC02

Mixed packing provisions (RID)	: MP15 Portable tank and bulk container instructions (RID)	: T11

Portable tank and bulk container special provisions (RID)

: TP2, TP27

Tank codes for RID tanks (RID)	: L4BN

Transport category (RID)	: 2

Colis express (express parcels) (RID)	: CE6

Hazard identification number (RID)	: 80

# Transport in bulk according to Annex II of Marpol and the IBC Code

Not applicable

SECTION 15: Regulatory information

# Safety, health and environmental regulations/legislation specific for the substance or mixture

## EU-Regulations

| EU restriction list (REACH Annex XVII) |  |  |
| --- | --- | --- |
| Reference code | Applicable on | Entry title or description |
| 3(b) | Quaternary ammonium compounds, benzyl-C12- 14 (even-numbered)- alkyldimethyl, chlorides | Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard classes 3.1 to 3.6, 3.7 adverse effects on sexual function and fertility or on development, 3.8 effects other than narcotic effects, 3.9 and 3.10 |
| 3(c) | Quaternary ammonium compounds, benzyl-C12- 14 (even-numbered)- alkyldimethyl, chlorides | Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard class 4.1 |

Contains no substance on the REACH candidate list Contains no REACH Annex XIV substances

Contains no substance subject to Regulation (EU) No 649/2012 of the European Parliament and of the Council of 4 July 2012 concerning the export and import of hazardous chemicals.

Contains no substance subject to Regulation (EU) No 2019/1021 of the European Parliament and of the Council of 20 June 2019 on persistent organic pollutants

## National regulations Germany

Employment restrictions	: Observe restrictions according Act on the Protection of Working Mothers (MuSchG) Observe restrictions according Act on the Protection of Young People in Employment (JArbSchG)

Water hazard class (WGK)	: WGK 3, Highly hazardous to water (Classification according to AwSV, Annex 1) Hazardous Incident Ordinance (12. BImSchV)	: Is not subject of the Hazardous Incident Ordinance (12. BImSchV)

## Netherlands

SZW-lijst van kankerverwekkende stoffen	: None of the components are listed SZW-lijst van mutagene stoffen	: None of the components are listed

NIET-limitatieve lijst van voor de voortplanting giftige stoffen – Borstvoeding

NIET-limitatieve lijst van voor de voortplanting giftige stoffen – Vruchtbaarheid

NIET-limitatieve lijst van voor de voortplanting giftige stoffen – Ontwikkeling

## Denmark

: None of the components are listed

: None of the components are listed

: None of the components are listed

Danish National Regulations	: Young people below the age of 18 years are not allowed to use the product

# Chemical safety assessment

No chemical safety assessment has been carried out

SECTION 16: Other information

| Full text of H- and EUH-statements |  |
| --- | --- |
| Acute Tox. 4 (Oral) | Acute toxicity (oral), Category 4 |
| Aquatic Acute 1 | Hazardous to the aquatic environment — Acute Hazard, Category 1 |
| Aquatic Chronic 1 | Hazardous to the aquatic environment — Chronic Hazard, Category 1 |

| Full text of H- and EUH-statements |  |
| --- | --- |
| Eye Dam. 1 | Serious eye damage/eye irritation, Category 1 |
| Skin Corr. 1B | Skin corrosion/irritation, Category 1, Sub-Category 1B |
| H302 | Harmful if swallowed. |
| H314 | Causes severe skin burns and eye damage. |
| H318 | Causes serious eye damage. |
| H400 | Very toxic to aquatic life. |
| H410 | Very toxic to aquatic life with long lasting effects. |

Safety Data Sheet (SDS), EU

This information is based on our current knowledge and is intended to describe the product for the purposes of health, safety and environmental requirements only. It s

"Emergency medical information: (seven days) contact National Poisons Information Centre, Beaumont Hospital, Dublin 9 DOV2NO, Ireland.

Members of the public Number (8 am-10 pm): "+353 (0)1 809 2166 ".

Healthcare professional telephone Number (24hrs): "+353 (0)1 809 2566"

hould not therefore be construed as guaranteeing any specific property of the product.