Kettle Boss

Safety Data Sheet

according to Regulation (EC) No. 1907/2006 (REACH) with its amendment Regulation (EU) 2015/830 Issue date: 3/13/2020 Version: 0.0

SECTION 1: Identification of the substance/mixture and of the company/undertaking

1.1. Product identifier

Product form	: Mixture

Product name	: Kettle Boss

EC-No.	: 200-579-1

CAS-No.	: 64-18-6

Product code	: KB02

1.2. Relevant identified uses of the substance or mixture and uses advised against

## Relevant identified uses

No additional information available

## Uses advised against

No additional information available

1.3. Details of the supplier of the safety data sheet

## Supplier

## Dargan Tools Limited

## Dolmen Business Park

## Tullow road

## Carlow

## (059) 9142855

1.4. Emergency telephone number

Emergency number	: +353 (086) 4067748

Office Hours: Monday to Thursday 9.00am- 5.30pm, Friday 9.00am- 4.30pm

| SECTION 2: Hazards identification |  |
| --- | --- |
| 2.1. Classification of the substance or mixture |  |
| Classification according to Regulation (EC) No. 1272/2008 [CLP] |  |
| Flammable liquids, Category 3 | H226 |
| Acute toxicity (oral), Category 4 | H302 |
| Acute toxicity (inhal.), Category 3 | H331 |
| Skin corrosion/irritation, Category 1, Sub-Category 1B | H314 |
| Serious eye damage/eye irritation, Category 1 | H318 |
| Hazardous to the aquatic environment — Chronic Hazard, Category 3 Full text of H statements : see section 16 | H412 |

## Adverse physicochemical, human health and environmental effects

Flammable liquid and vapour. Toxic if inhaled. Harmful if swallowed. Causes severe skin burns and eye damage. Causes serious eye damage. Harmful to aquatic life with long lasting effects.

2.2. Label elements

## Labelling according to Regulation (EC) No. 1272/2008 [CLP]

Hazard pictograms (CLP)	:

GHS02	GHS05	GHS06

Signal word (CLP)	: Danger

Hazardous ingredients	: Kettle Boss

Hazard statements (CLP)	: H226 - Flammable liquid and vapour.

H302 - Harmful if swallowed.

H314 - Causes severe skin burns and eye damage. H331 - Toxic if inhaled.

H412 - Harmful to aquatic life with long lasting effects.

Precautionary statements (CLP)	: P210 - Keep away from sparks, hot surfaces, heat, open flames. No smoking.

P241 - Use explosion-proof electrical, lighting, ventilating equipment. P260 - Do not breathe fume, mist, spray, vapours.

P280 - Wear protective clothing, eye protection, face protection, protective gloves. P301+P312 - IF SWALLOWED: Call doctor, a POISON CENTER if you feel unwell. P301+P330+P331+P310 - IF SWALLOWED: rinse mouth. Do NOT induce vomiting. Immediately call a doctor, a POISON CENTER.

2.3. Other hazards

No additional information available

SECTION 3: Composition/information on ingredients

3.1. Substances

Not applicable

3.2. Mixtures

| Name | Product identifier | % | Classification according to Regulation (EC) No. 1272/2008 [CLP] |
| --- | --- | --- | --- |
| Formic Acid | (CAS-No.) 64-18-6 (EC-No.) 200-579-1 | 33 - 66 | Flam. Liq. 3, H226 Acute Tox. 4 (Oral), H302 Acute Tox. 3 (Inhalation), H331 Skin Corr. 1A, H314 Eye Dam. 1, H318 |

Full text of H-statements: see section 16

SECTION 4: First aid measures

4.1. Description of first aid measures

First-aid measures general	: Call a physician immediately.

First-aid measures after inhalation	: Remove person to fresh air and keep comfortable for breathing. Call a doctor.

First-aid measures after skin contact	: Rinse skin with water/shower. Take off immediately all contaminated clothing. Call a

physician immediately.

First-aid measures after eye contact	: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy

to do. Continue rinsing. Call a physician immediately.

First-aid measures after ingestion	: Rinse mouth. Do not induce vomiting. Give 500 ml water to drink. Do not give an

unconscious person anything to drink. Call a physician immediately.

4.2. Most important symptoms and effects, both acute and delayed

Symptoms/effects after inhalation	: May cause irritation to the respiratory tract.

Symptoms/effects after skin contact	: Burns.

Symptoms/effects after eye contact	: Serious damage to eyes.

Symptoms/effects after ingestion	: Burns. Ingestion may cause nausea and vomiting.

4.3. Indication of any immediate medical attention and special treatment needed

Treat symptomatically.

SECTION 5: Firefighting measures

5.1. Extinguishing media

Suitable extinguishing media	: Alcohol-resistant foam. Carbon dioxide. Dry powder. Use water spray or fog for cooling

exposed containers.

5.2. Special hazards arising from the substance or mixture

Fire hazard	: Flammable liquid and vapour.

Explosion hazard	: May form flammable/explosive vapour-air mixture. Hazardous decomposition products in case of fire	: Toxic fumes may be released.

5.3. Advice for firefighters

Protection during firefighting	: Do not attempt to take action without suitable protective equipment. Self-contained

breathing apparatus. Complete protective clothing.

SECTION 6: Accidental release measures

6.1. Personal precautions, protective equipment and emergency procedures

General measures	: Turn leaking containers leak-side up to prevent the escape of liquid. Eliminate every possible source of ignition.

## For non-emergency personnel

Emergency procedures	: Ventilate spillage area. No open flames, no sparks, and no smoking. Avoid contact with skin and eyes. Do not breathe fume, mist, spray, vapours. Keep upwind.

## For emergency responders

Protective equipment	: Do not attempt to take action without suitable protective equipment. For further information refer to section 8: "Exposure controls/personal protection".

6.2. Environmental precautions

Do not discharge into drains or rivers. Contain the spilled material by bunding.

6.3. Methods and material for containment and cleaning up

Methods for cleaning up	: Use non-sparking tools. Absorb excess liquid spillage on inorganic adsorbent material such as fine sand, brick dust etc. Place spent adsorbent in sealed packages and contact specialist waste disposal contractor.

Other information	: Dispose of materials or solid residues at an authorized site.

6.4. Reference to other sections

For further information refer to section 13.

SECTION 7: Handling and storage

7.1. Precautions for safe handling

Precautions for safe handling	: Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No

smoking. Ground/bond container and receiving equipment. Use only non-sparking tools. Take precautionary measures against static discharge. Flammable vapours may accumulate in the container. Use explosion-proof equipment. Wear personal protective equipment. Use only outdoors or in a well-ventilated area. Avoid contact with skin and eyes. Do not breathe fume, mist, spray, vapours.

Hygiene measures	: Wash contaminated clothing before reuse. Do not eat, drink or smoke when using this product. Always wash hands after handling the product.

7.2. Conditions for safe storage, including any incompatibilities

Technical measures	: Ground/bond container and receiving equipment. Prevent the build-up of electrostatic charge.

Storage conditions	: Store in a well-ventilated place. Keep cool. Keep container tightly closed. Store locked up.

Heat and ignition sources	: Keep away from ignition sources (including static discharges).

7.3. Specific end use(s)

No additional information available

SECTION 8: Exposure controls/personal protection

8.1. Control parameters

| Formic Acid (64-18-6) |  |
| --- | --- |
| EU - Occupational Exposure Limits |  |
| IOELV TWA (mg/m³) | 9 mg/m³ |
| IOELV TWA (ppm) | 5 ppm |
| United Kingdom - Occupational Exposure Limits |  |
| WEL TWA (mg/m³) | 9.6 mg/m³ |
| WEL TWA (ppm) | 5 ppm |

8.2. Exposure controls

## Appropriate engineering controls:

Ensure good ventilation of the work station. Use spark-/explosionproof appliances and lighting system.

Impermeable protective gloves

Hand protection:

Chemical goggles or safety glasses. Emergency eye wash fountains should be available in the immediate vicinity of any potential exposure

Eye protection:

Wear suitable protective clothing

Skin and body protection:

[In case of inadequate ventilation] wear respiratory protection.

Respiratory protection:

## Personal protective equipment symbol(s):

Environmental exposure controls:

Avoid release to the environment.

SECTION 9: Physical and chemical properties

9.1. Information on basic physical and chemical properties

Physical state	: Liquid

Colour	: Colourless.

Odour	: characteristic.

Odour threshold	: No data available

pH	: 2

Relative evaporation rate (butylacetate=1)	: No data available

Melting point	: -13 °C Not applicable

Freezing point	: No data available

Boiling point	: 107.3 °C

Flash point	: > 60 °C

Auto-ignition temperature	: 528 °C

Decomposition temperature	: No data available

Flammability (solid, gas)	: Not applicable

Vapour pressure	: 24.2 hPa

Relative vapour density at 20 °C	: No data available

Relative density	: 1.195

Solubility	: Water: Miscible

Partition coefficient n-octanol/water (Log Pow)	: No data available Partition coefficient n-octanol/water (Log Kow)	: ≥ -2.1

Viscosity, kinematic	: No data available

Viscosity, dynamic	: No data available

Explosive properties	: No data available

Oxidising properties	: No data available

Explosive limits	: No data available

9.2. Other information

No additional information available

SECTION 10: Stability and reactivity

10.1. Reactivity

Flammable liquid and vapour.

10.2. Chemical stability

Stable under normal conditions.

10.3. Possibility of hazardous reactions

No dangerous reactions known under normal conditions of use.

10.4. Conditions to avoid

Avoid contact with hot surfaces. Heat. No flames, no sparks. Eliminate all sources of ignition.

10.5. Incompatible materials

Strong oxidizing agents. Strong acids.

10.6. Hazardous decomposition products

Thermal decomposition generates toxic vapours.

SECTION 11: Toxicological information

11.1. Information on toxicological effects

Acute toxicity (oral)	: Harmful if swallowed.

Acute toxicity (dermal)	: Not classified

Acute toxicity (inhalation)	: Toxic if inhaled.

| Formic Acid 85% (64-18-6) |  |
| --- | --- |
| ATE CLP (oral) | 811.111 mg/kg bodyweight |
| ATE CLP (gases) | 700 ppmv/4h |
| ATE CLP (vapours) | 3 mg/l/4h |
| ATE CLP (dust,mist) | 0.5 mg/l/4h |

| Kettle Boss |  |
| --- | --- |
| LD50 oral rat | 730 mg/kg bodyweight Animal: rat, Guideline: OECD Guideline 401 (Acute Oral Toxicity), 95% CL: 618 - 863 |
| LD50 dermal rat | > 2000 mg/kg bodyweight Animal: rat, Guideline: OECD Guideline 402 (Acute Dermal Toxicity) |
| LC50 Inhalation - Rat | 7.85 mg/l air Animal: rat, Guideline: OECD Guideline 403 (Acute Inhalation Toxicity) |

Skin corrosion/irritation	: Causes severe skin burns. pH: 2

Serious eye damage/irritation	: Causes serious eye damage. pH: 2

Respiratory or skin sensitisation	: Not classified

Germ cell mutagenicity	: Not classified

Carcinogenicity	: Not classified

| Kettle Boss |  |
| --- | --- |
| NOAEL (chronic, oral, animal/male, 2 years) | 400 mg/kg bodyweight Animal: mouse, Animal sex: male, Guideline: OECD Guideline 453 (Combined Chronic Toxicity / Carcinogenicity Studies), Remarks on results: other:Effect type: toxicity (migrated information) |

Reproductive toxicity	: Not classified

STOT-single exposure	: Not classified

STOT-repeated exposure	: Not classified

| Kettle Boss |  |
| --- | --- |
| LOAEL (oral, rat, 90 days) | 2000 mg/kg bodyweight Animal: rat, Guideline: OECD Guideline 453 (Combined Chronic Toxicity / Carcinogenicity Studies) |
| NOAEL (oral, rat, 90 days) | 400 mg/kg bodyweight Animal: rat, Guideline: OECD Guideline 453 (Combined Chronic Toxicity / Carcinogenicity Studies) |
| NOAEC (inhalation, rat, dust/mist/fume, 90 days) | 0.244 mg/l air Animal: rat, Guideline: OECD Guideline 413 (Subchronic Inhalation Toxicity: 90-Day Study) |

Aspiration hazard	: Not classified

SECTION 12: Ecological information

# Toxicity

Ecology - general	: Harmful to aquatic life with long lasting effects.

Hazardous to the aquatic environment, short-term (acute)

Hazardous to the aquatic environment, long-term (chronic)

Not rapidly degradable

: Not classified

: Harmful to aquatic life with long lasting effects.

| Kettle Boss |  |
| --- | --- |
| LC50 fish 1 | 68 mg/l Leuciswcus idus, 96 h |
| EC50 Daphnia 1 | 1.24 mg/l Daphnia magna, 48 h |
| EC50 72h algae (1) | 1240 mg/l Test organisms (species): Pseudokirchneriella subcapitata (previous names: Raphidocelis subcapitata, Selenastrum capricornutum) |
| LOEC (chronic) | > 100 mg/l Test organisms (species): Daphnia magna Duration: '21 d' |
| NOEC (chronic) | ≥ 100 mg/l Test organisms (species): Daphnia magna Duration: '21 d' |

  - Persistence and degradability

| Kettle Boss |  |
| --- | --- |
| Persistence and degradability | Product is biodegradable. |

  - Bioaccumulative potential

| Kettle Boss |  |
| --- | --- |
| Partition coefficient n-octanol/water (Log Kow) | ≥ -2.1 |
| Bioaccumulative potential | No bioaccumulation. |

  - Mobility in soil

| Kettle Boss |  |
| --- | --- |
| Ecology - soil | Adsorbs into the soil. |

  - Results of PBT and vPvB assessment

| Kettle Boss |  |
| --- | --- |
| Results of PBT assessment | This product is not identified as a PBT/vPvB substance. |

  - Other adverse effects

Other adverse effects	: Ecotoxicity: negligible.

SECTION 13: Disposal considerations

# 13.1. Waste treatment methods

Waste treatment methods	: Dispose of contents/container in accordance with licensed collector’s sorting instructions.

Must follow special treatment according to local regulation.

Additional information	: Flammable vapours may accumulate in the container.

SECTION 14: Transport information

In accordance with ADR / RID / IMDG / IATA / ADN

| ADR | IMDG | IATA | ADN | RID |
| --- | --- | --- | --- | --- |
| 14.1. UN number |  |  |  |  |
| UN 3412 | UN 3412 | UN 3412 | UN 3412 | UN 3412 |
| 14.2. UN proper shipping name |  |  |  |  |
| FORMIC ACID | FORMIC ACID | Formic acid | FORMIC ACID | FORMIC ACID |
| Transport document description |  |  |  |  |
| UN 3412 FORMIC ACID, 8, II, (E) | UN 3412 FORMIC ACID, 8, II | UN 3412 Formic acid, 8, II | UN 3412 FORMIC ACID, 8, II | UN 3412 FORMIC ACID, 8, II |
| 14.3. Transport hazard class(es) |  |  |  |  |
| 8 | 8 | 8 | 8 | 8 |

| 14.4. Packing group |  |  |  |  |
| --- | --- | --- | --- | --- |
| II | II | II | II | II |
| 14.5. Environmental hazards |  |  |  |  |
| Dangerous for the environment : No | Dangerous for the environment : No Marine pollutant : No | Dangerous for the environment : No | Dangerous for the environment : No | Dangerous for the environment : No |
| No supplementary information available |  |  |  |  |

# Special precautions for user

## Overland transport

Classification code (ADR)	: C3

Limited quantities (ADR)	: 1l

Excepted quantities (ADR)	: E2

Packing instructions (ADR)	: P001, IBC02

Mixed packing provisions (ADR)	: MP15 Portable tank and bulk container instructions (ADR) : T7

Portable tank and bulk container special provisions (ADR)

: TP2

Tank code (ADR)	: L4BN

Vehicle for tank carriage	: AT

Transport category (ADR)	: 2 Hazard identification number (Kemler No.)	: 80 Orange plates	:

Tunnel restriction code (ADR)	: E

EAC code	: •2X

## Transport by sea

Packing instructions (IMDG)	: P001

IBC packing instructions (IMDG)	: IBC02

Tank instructions (IMDG)	: T7

Tank special provisions (IMDG)	: TP2

EmS-No. (Fire)	: F-A

EmS-No. (Spillage)	: S-B

Stowage category (IMDG)	: A

Stowage and handling (IMDG)	: SW2

Properties and observations (IMDG)	: Colourless liquid with a pungent odour. Corrosive to most metals. Causes burns to skin, eyes and mucous membranes.

## Air transport

PCA Excepted quantities (IATA)	: E2

PCA Limited quantities (IATA)	: Y840 PCA limited quantity max net quantity (IATA)	: 0.5L PCA packing instructions (IATA)	: 851

PCA max net quantity (IATA)	: 1L

CAO packing instructions (IATA)	: 855

CAO max net quantity (IATA)	: 30L

ERG code (IATA)	: 8L

## Inland waterway transport

Classification code (ADN)	: C3

Limited quantities (ADN)	: 1 L

Excepted quantities (ADN)	: E2

Equipment required (ADN)	: PP, EP

Number of blue cones/lights (ADN)	: 0

## Rail transport

Classification code (RID)	: C3

Limited quantities (RID)	: 1L

Excepted quantities (RID)	: E2

Packing instructions (RID)	: P001, IBC02

Mixed packing provisions (RID)	: MP15 Portable tank and bulk container instructions (RID)	: T7

Portable tank and bulk container special provisions (RID)

: TP2

Tank codes for RID tanks (RID)	: L4BN

Transport category (RID)	: 2

Colis express (express parcels) (RID)	: CE6

Hazard identification number (RID)	: 80

# Transport in bulk according to Annex II of Marpol and the IBC Code

Not applicable

SECTION 15: Regulatory information

# Safety, health and environmental regulations/legislation specific for the substance or mixture

## EU-Regulations

| The following restrictions are applicable according to Annex XVII of the REACH Regulation (EC) No 1907/2006: |  |  |
| --- | --- | --- |
| Reference code | Applicable on | Entry title or description |
| 3(a) | Formic Acid 85% ; Formic Acid | Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard classes 2.1 to 2.4, 2.6 and 2.7, 2.8 types A and B, 2.9, 2.10, 2.12, 2.13 categories 1 and 2, 2.14 categories 1 and 2, 2.15 types A to F |
| 3(b) | Formic Acid 85% ; Formic Acid | Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard classes 3.1 to 3.6, 3.7 adverse effects on sexual function and fertility or on development, 3.8 effects other than narcotic effects, 3.9 and 3.10 |
| 3(c) | Formic Acid 85% | Substances or mixtures fulfilling the criteria for any of the following hazard classes or categories set out in Annex I to Regulation (EC) No 1272/2008: Hazard class 4.1 |
| 40. | Formic Acid 85% ; Formic Acid | Substances classified as flammable gases category 1 or 2, flammable liquids categories 1, 2 or 3, flammable solids category 1 or 2, substances and mixtures which, in contact with water, emit flammable gases, category 1, 2 or 3, pyrophoric liquids category 1 or pyrophoric solids category 1, regardless of whether they appear in Part 3 of Annex VI to Regulation (EC) No 1272/2008 or not. |

Contains no substance on the REACH candidate list Contains no REACH Annex XIV substances

Contains no substance subject to Regulation (EU) No 649/2012 of the European Parliament and of the Council of 4 July 2012 concerning the export and import of hazardous chemicals.

Contains no substance subject to Regulation (EU) No 2019/1021 of the European Parliament and of the Council of 20 June 2019 on persistent organic pollutants

## National regulations Germany

Employment restrictions	: Observe restrictions according Act on the Protection of Working Mothers (MuSchG) Observe restrictions according Act on the Protection of Young People in Employment (JArbSchG)

Water hazard class (WGK)	: WGK 1, Slightly hazardous to water (Classification according to AwSV, Annex 1) Hazardous Incident Ordinance (12. BImSchV)	: Is not subject of the Hazardous Incident Ordinance (12. BImSchV)

## Netherlands

SZW-lijst van kankerverwekkende stoffen	: None of the components are listed SZW-lijst van mutagene stoffen	: None of the components are listed

NIET-limitatieve lijst van voor de voortplanting giftige stoffen – Borstvoeding

NIET-limitatieve lijst van voor de voortplanting giftige stoffen – Vruchtbaarheid

NIET-limitatieve lijst van voor de voortplanting giftige stoffen – Ontwikkeling

: None of the components are listed

: None of the components are listed

: None of the components are listed

## Denmark

Classification remarks	: Emergency management guidelines for the storage of flammable liquids must be followed Danish National Regulations	: Young people below the age of 18 years are not allowed to use the product

# Chemical safety assessment

No chemical safety assessment has been carried out

SECTION 16: Other information

| Abbreviations and acronyms: |  |
| --- | --- |
| ADN | European Agreement concerning the International Carriage of Dangerous Goods by Inland Waterways |
| ADR | European Agreement concerning the International Carriage of Dangerous Goods by Road |
| ATE | Acute Toxicity Estimate |
| BLV | Biological limit value |
| CAS-No. | Chemical Abstract Service number |
| CLP | Classification Labelling Packaging Regulation; Regulation (EC) No 1272/2008 |
| DMEL | Derived Minimal Effect level |
| DNEL | Derived-No Effect Level |
| EC50 | Median effective concentration |
| EC-No. | European Community number |
| EN | European Standard |
| IATA | International Air Transport Association |
| IMDG | International Maritime Dangerous Goods |
| LC50 | Median lethal concentration |
| LD50 | Median lethal dose |
| LOAEL | Lowest Observed Adverse Effect Level |
| NOAEC | No-Observed Adverse Effect Concentration |
| NOAEL | No-Observed Adverse Effect Level |
| NOEC | No-Observed Effect Concentration |
| OEL | Occupational Exposure Limit |
| PBT | Persistent Bioaccumulative Toxic |
| PNEC | Predicted No-Effect Concentration |
| REACH | Registration, Evaluation, Authorisation and Restriction of Chemicals Regulation (EC) No 1907/2006 |
| RID | Regulations concerning the International Carriage of Dangerous Goods by Rail |
| SDS | Safety Data Sheet |
| vPvB | Very Persistent and Very Bioaccumulative |
| WGK | Water Hazard Class |

| Full text of H- and EUH-statements: |  |
| --- | --- |
| Acute Tox. 3 (Inhalation) | Acute toxicity (inhal.), Category 3 |
| Acute Tox. 4 (Oral) | Acute toxicity (oral), Category 4 |
| Eye Dam. 1 | Serious eye damage/eye irritation, Category 1 |
| Flam. Liq. 3 | Flammable liquids, Category 3 |

| Skin Corr. 1A | Skin corrosion/irritation, Category 1, Sub-Category 1A |
| --- | --- |
| H226 | Flammable liquid and vapour. |
| H302 | Harmful if swallowed. |
| H314 | Causes severe skin burns and eye damage. |
| H318 | Causes serious eye damage. |
| H331 | Toxic if inhaled. |
| H412 | Harmful to aquatic life with long lasting effects. |

SDS EU (REACH Annex II)

This information is based on our current knowledge and is intended to describe the product for the purposes of health, safety and environmental requirements only. It should not therefore be construed as guaranteeing any specific property of the product.

"Emergency medical information: (seven days) contact National Poisons Information Centre, Beaumont Hospital, Dublin 9 DOV2NO, Ireland.

Members of the public Number (8 am-10 pm): "+353 (0)1 809 2166 ".

Healthcare professional telephone Number (24hrs): "+353 (0)1 809 2566"